CIVL 1100 Record Sheets for Structural Engineering Experiments

Table 1.  Experimental data for Three Pinned Arch Test

*Percentage error = (Experimental–Theoretical)/Theoretical × 100%

^Correct up to 3 decimal places

| Mass | Distance from left | Span Fraction | Experimental horizontal reaction | Theoretical horizontal reaction | Theoretical vertical reaction (at support A) | *Percentage error in horizontal reaction^ |
| --- | --- | --- | --- | --- | --- | --- |
| g | mm | -- | N | N | N | -- |
| 300 | 0 | 0 | 0.0 | 0.00 | 2.94 | -- |
| 300 | 50 | 0.1 | 0.7 | 0.74 | 2.65 | -5.405% |
| 300 | 100 | 0.2 | 1.6 | 1.47 | 2.35 | 8.844% |
| 300 | 150 | 0.3 | 2.2 | 2.21 | 2.06 | -0.452% |
| 300 | 200 | 0.4 | 2.9 | 2.94 | 1.77 | -1.361% |
| 300 | 250 | 0.5 | 3.2 | 3.68 | 1.47 | -13.043% |
| 300 | 300 | 0.6 | 2.9 | 2.94 | 1.18 | -1.361% |
| 300 | 350 | 0.7 | 2.2 | 2.21 | 0.88 | -0.452% |
| 300 | 400 | 0.8 | 1.6 | 1.47 | 0.59 | 8.844% |
| 300 | 450 | 0.9 | 0.9 | 0.74 | 0.29 | 21.622% |
| 300 | 500 | 1 | 0.2 | 0.00 | 0.00 | -- |

Table 2.  Experimental data for Redundant Truss Test (Statically Determinate Structure)

Net strain (ε) = Measured strain at different force level - Measured strain when Force = 0 N (initial strain)

*Actual force (F) = Young’s modulus (E) × Cross-sectional area of truss member (A) × Net strain (ε)

# Negative net strain and force represent compression & Member not used ^ Member sensor is broken, theoretical value is used instead

For Force = 0 N

| Member | 1 | 2^ | 3 | 4 | 5 | 6& | 7^ | 8^ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured strain (με) | 0 | (0) | 0 | 0 | 0 | -- | (0) | (0) |
| Theoretical strain (με) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -- | 0.0 | 0.0 |

For Force = 50 N

| Member | 1 | 2^ | 3 | 4 | 5 | 6& | 7^ | 8^ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured strain (με) | 10 | (-8.4) | -9 | -17 | 0 | -- | (11.9) | (11.9) |
| *Actual force (N) | 62 | (-50.0) | -54 | -99 | 2 | -- | (70.7) | (70.7) |
| Theoretical strain (με) | 8.4 | -8.4 | -8.4 | -16.8 | 0.0 | -- | 11.9 | 11.9 |
| Theoretical force (N) | 50.0 | -50.0 | -50.0 | -100.0 | 0.0 | -- | 70.7 | 70.7 |
| Displacement (mm) | 0.023 (Hardware test)@ |  |  |  | 0.012 (Virtual experiment)@ |  |  |  |

For Force = 100 N

| Member | 1 | 2^ | 3 | 4 | 5 | 6& | 7^ | 8^ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured strain (με) | 21 | (-16.8) | -18 | -35 | 1 | -- | (23.8) | (23.8) |
| *Actual force (N) | 124 | (-100.0) | -105 | -211 | 4 | -- | (141.4) | (141.4) |
| Theoretical strain (με) | 16.8 | -16.8 | -16.8 | -33.7 | 0.0 | -- | 23.8 | 23.8 |
| Theoretical force (N) | 100.0 | -100.0 | -100.0 | -200.0 | 0.0 | -- | 141.4 | 141.4 |
| Displacement (mm) | 0.045 (Hardware test)@ |  |  |  | 0.024 (Virtual experiment)@ |  |  |  |

@ Note: Displacement in virtual experiments is given in downward displacement; both displacements are corrected so that positive is downward displacement

Table 2. (cont’d)		Experimental data for Redundant Truss Test (Statically Determinate Structure)

For Force = 150 N

| Member | 1 | 2^ | 3 | 4 | 5 | 6& | 7^ | 8^ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured strain (με) | 31 | (-25.3) | -27 | -55 | 1 | -- | (35.7) | (35.7) |
| *Actual force (N) | 181 | (-150.0) | -163 | -325 | 5 | -- | (212.1) | (212.1) |
| Theoretical strain (με) | 25.3 | -25.3 | -25.3 | -50.5 | 0.0 | -- | 35.7 | 35.7 |
| Theoretical force (N) | 150.0 | -150.0 | -150.0 | -300.0 | 0.0 | -- | 212.1 | 212.1 |
| Displacement (mm) | 0.064 (Hardware test)@ |  |  |  | 0.035 (Virtual experiment)@ |  |  |  |

For Force = 200 N

| Member | 1 | 2^ | 3 | 4 | 5 | 6& | 7^ | 8^ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured strain (με) | 41 | (-33.7) | -37 | -72 | 1 | -- | (47.6) | (47.6) |
| *Actual force (N) | 243 | (-200.0) | -221 | -430 | 4 | -- | (282.8) | (282.8) |
| Theoretical strain (με) | 33.7 | -33.7 | -33.7 | -67.4 | 0.0 | -- | 47.6 | 47.6 |
| Theoretical force (N) | 200.0 | -200.0 | -200.0 | -400.0 | 0.0 | -- | 282.8 | 282.8 |
| Displacement (mm) | 0.081 (Hardware test)@ |  |  |  | 0.047 (Virtual experiment)@ |  |  |  |

For Force = 250 N (NOT required because the experiment equipment cannot handle 250 N)

| Member | 1 | 2^ | 3 | 4 | 5 | 6& | 7^ | 8^ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured strain (με) | -- | -- | -- | -- | -- | -- | -- | -- |
| *Actual force (N) | -- | -- | -- | -- | -- | -- | -- | -- |
| Theoretical strain (με) | -- | -- | -- | -- | -- | -- | -- | -- |
| Theoretical force (N) | -- | -- | -- | -- | -- | -- | -- | -- |
| Displacement (mm) | -- (Hardware test)@ |  |  |  | -- (Virtual experiment)@ |  |  |  |

Table 3.  Experimental data for Redundant Truss Test (Statically Indeterminate Structure)

For Force = 0 N

| Member | 1 | 2^ | 3 | 4 | 5 | 6 | 7^ | 8^ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured strain (με) | 0 | (0) | 0 | 0 | 0 | 0 | (0) | (0) |
| Theoretical strain (με) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

For Force = 50 N

| Member | 1 | 2^ | 3 | 4 | 5 | 6 | 7^ | 8^ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured strain (με) | 13 | (-4.2) | -9 | -14 | 3 | -3 | (11.9) | (6.0) |
| *Actual force (N) | 79 | (-25.0) | -52 | -85 | 19 | -20 | (70.7) | (35.4) |
| Theoretical strain (με) | 12.6 | -4.2 | -8.4 | -12.6 | 4.2 | -6.0 | 11.9 | 6.0 |
| Theoretical force (N) | 75.0 | -25.0 | -50.0 | -75.0 | 25.0 | -35.4 | 70.7 | 35.4 |
| Displacement (mm) | 0.020 (Hardware test)@ |  |  |  | 0.007 (Virtual experiment)@ |  |  |  |

For Force = 100 N

| Member | 1 | 2^ | 3 | 4 | 5 | 6 | 7^ | 8^ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured strain (με) | 27 | (-8.4) | -19 | -30 | 7 | -8 | (23.8) | (11.9) |
| *Actual force (N) | 162 | (-50.0) | -110 | -176 | 40 | -49 | (141.4) | (70.7) |
| Theoretical strain (με) | 25.3 | -8.4 | -16.8 | -25.3 | 8.4 | -11.9 | 23.8 | 11.9 |
| Theoretical force (N) | 150.0 | -50.0 | -100.0 | -150.0 | 50.0 | -70.7 | 141.4 | 70.7 |
| Displacement (mm) | 0.037 (Hardware test)@ |  |  |  | 0.014 (Virtual experiment)@ |  |  |  |

Table 3. (cont’d)		Experimental data for Redundant Truss Test (Statically Indeterminate Structure)

For Force = 150 N

| Member | 1 | 2^ | 3 | 4 | 5 | 6 | 7^ | 8^ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured strain (με) | 41 | (-12.6) | -28 | -44 | 10 | -14 | (35.7) | (17.9) |
| *Actual force (N) | 241 | (-75.0) | -167 | -264 | 61 | -80 | (212.1) | (106.1) |
| Theoretical strain (με) | 37.9 | -12.6 | -25.3 | -37.9 | 12.6 | -17.9 | 35.7 | 17.9 |
| Theoretical force (N) | 225.0 | -75.0 | -150.0 | -225.0 | 75.0 | -106.1 | 212.1 | 106.1 |
| Displacement (mm) | 0.049 (Hardware test)@ |  |  |  | 0.021 (Virtual experiment)@ |  |  |  |

For Force = 200 N

| Member | 1 | 2^ | 3 | 4 | 5 | 6 | 7^ | 8^ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured strain (με) | 55 | (-16.8) | -38 | -58 | 15 | -19 | (47.6) | (23.8) |
| *Actual force (N) | 327 | (-100.0) | -223 | -344 | 88 | -113 | (282.8) | (141.4) |
| Theoretical strain (με) | 50.5 | -16.8 | -33.7 | -50.5 | 16.8 | -23.8 | 47.6 | 23.8 |
| Theoretical force (N) | 300.0 | -100.0 | -200.0 | -300.0 | 100.0 | -141.4 | 282.8 | 141.4 |
| Displacement (mm) | 0.062 (Hardware test)@ |  |  |  | 0.028 (Virtual experiment)@ |  |  |  |

For Force = 250 N (NOT required because the experiment equipment cannot handle 250 N)

| Member | 1 | 2^ | 3 | 4 | 5 | 6 | 7^ | 8^ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measured strain (με) | -- | -- | -- | -- | -- | -- | -- | -- |
| *Actual force (N) | -- | -- | -- | -- | -- | -- | -- | -- |
| Theoretical strain (με) | -- | -- | -- | -- | -- | -- | -- | -- |
| Theoretical force (N) | -- | -- | -- | -- | -- | -- | -- | -- |
| Displacement (mm) | -- (Hardware test)@ |  |  |  | -- (Virtual experiment)@ |  |  |  |