ELEC 1100 Laboratory 2: Summary Sheet

Name: __________________________        Student ID:

Notes:

(1) Show demo to your TA for an attendance record; bonus mark will be given to groups who can complete all TA Checks in the first 2 hours of your enrolled lab session.

LA2 (Fri, before 15:30), LA3 (Mon, before 12:30), LA1 (Tue, before 15:30)

(2) Individually submit your completed lab summary sheet to Canvas (LA1/LA2/LA3/LA4) “Assignments” before the deadline.

LA2 (Fri, deadline 16:20), LA3 (Mon, deadline 13:20), LA1 (Tue, deadline 16:20)

Experimental Results and Analysis

Q1: Calculate for Req and complete the table below.

Q2: Record the value of Req displayed on the screen (including the unit Ω/kΩ) to fill in the table below.

| Figure 1 | Figure 2 |
| --- | --- |
| B A 2kΩ 1kΩ Req | B A 2kΩ 2kΩ 1.5kΩ 1kΩ Req |
| By calculation: Req = (show your calculation steps) | By calculation: Req = (show your calculation steps) |
| From the multimeter: Req = This value should match your calculation above. | From the multimeter: Req = This value should match your calculation above. |

1.5R

R

2R

1.5R

1.5R

1.5R

Req

2R

2R

2R

2R

……

Q3: Based on what you got above, analysis a resistor network with infinite modules (R, 1.5R, 2R) as below. Prove that Req = 3R. Show clearly the calculation steps.

Q4: Record the measurement of voltages VAD, VAB, VBC, VCD and VDA to fill in the table below.

Q5: Complete the table to show that the sum of the voltages ACROSS all of the components (including the power source) in a loop complies with Kirchhoff’s voltage law (KVL).

| VAD | VAB | VBC | VCD | VDA |
| --- | --- | --- | --- | --- |
| VAD = VAB + VBC + VCD (use the measured voltages to verify the equation) VAD = VAB + VBC + VCD = |  |  |  |  |
| VAB + VBC + VCD + VDA = 0 (use the measured voltages to verify the equation) VAB + VBC + VCD + VDA = |  |  |  |  |

Q6: Calculate for branch currents I1, I2 and I3 to fill in the table below.

Q7: Record the value of I displayed on the screen (including the unit “mA”). Complete the table to show that the sum of currents complies with Kirchhoff’s current law (KCL).

I = I1 + I2 + I3

By calculation: I1 = ______________ I2 = ______________ I3 = ______________ I1 + I2 + I3 =

From the multimeter: Total current I = This value should match your calculation above.

I1

10V

6V

2kΩ

1kΩ

2kΩ

+

-

+

-

X

I

I2

Q8: Use KCL to solve the currents I, I1 and I2, then determine the voltage at node X in the cirucit below.

Show your calculation steps. **Notice the unit and direction of each value.

I = ___________      I1 = ___________      I2 = __________      VX = ___________

Q9: Record the value of VX from the multimeter to fill in the table below.

Q10: Record the value of I from the multimeter (including the unit “mA”) to fill in the table below.

| From the multimeter: VX = This value should match your calculated VX in Q8. | From the multimeter: I = This value should match your calculated I in Q8. |
| --- | --- |

Q11: Record the value of Req from the multimeter (including the unit Ω/kΩ) to fill in the table.

From the multimeter: Req =

Q12: Record the voltages from the multimeter and fill in the table.

| From the multimeter: VC = (versus GND) | From the multimeter: VD = (versus GND) |
| --- | --- |

Q13: Record the value of I from the multimeter (including the unit “mA”) to fill in the table.

From the multimeter: I =

Q14: Based on your record in Q13, use Ohm’s law to calculate for Req to fill in the table.

By calculation (including the unit Ω/kΩ): Req=VI= This value should match your measured Req in Q11.